# Magnitude comparator

A magnitude comparator is a digital circuit that compares the magnitudes (sizes) of two binary numbers and outputs the result of the comparison. It takes two binary inputs (say, A and B) and compares their values to determine the following:

- A > B (A is greater than B)

- A < B (A is less than B)

- A = B (A is equal to B)

2-bit Magnitude Comparator

A comparator that compares two binary numbers (each number having 2 bits) and produces three outputs based on the relative magnitudes of given binary bits is called a 2-bit magnitude comparator.

We're going to compare two numbers, let’s call them A and B. Now, since these are 2-bit numbers, they each have two binary digits. So A will be represented by A1 and A0, and B will be represented by B1 and B0. A1 and B1 are the most significant bits (MSBs), and A0 and B0 are the least significant bits (LSBs).

B

A

| A1 | A0 | B1 | B0 | A<B | A=B | A>B |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| 0 | 0 | 0 | 1 | 1 | 0 | 0 |
| 0 | 0 | 1 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 | 0 | 0 |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 | 1 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 | 0 | 1 |
| 1 | 0 | 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 1 | 1 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 | 0 | 1 |
| 1 | 1 | 0 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 0 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 | 1 | 0 |